CODE FOR1:  -

const int sensorPin = 2;

const int greenLED = 8;

const int redLED = 9;

const unsigned long TIMEOUT = 3000;

void setup() {

pinMode(sensorPin, INPUT_PULLUP);

pinMode(greenLED, OUTPUT);

pinMode(redLED, OUTPUT);

Serial.begin(9600);

Serial.println("=== Sensor Timeout Simulation ===");

}

void loop() {

Serial.println();

Serial.println("Requesting Sensor Data...");

unsigned long startTime = millis();

bool sensorReceived = false;

while (millis() - startTime < TIMEOUT) {

if (digitalRead(sensorPin) == LOW) {

sensorReceived = true;

break;

}

}

if (sensorReceived) {

Serial.println("Sensor Response Received");

digitalWrite(greenLED, HIGH);

digitalWrite(redLED, LOW);

delay(1000);

digitalWrite(greenLED, LOW);

}

else {

Serial.println("ERROR : Sensor Communication Timeout");

digitalWrite(redLED, HIGH);

delay(1000);

digitalWrite(redLED, LOW);

}

delay(1000);

}                                    SIMULATION :-

CODE FOR: - 2

#include <avr/io.h>

#include <avr/wdt.h>

const int GREEN_LED = 8;

const int RED_LED = 9;

unsigned long startTime;

bool failureTriggered = false;

void setup() {

// Disable Watchdog after reset

MCUSR = 0;

wdt_disable();

pinMode(GREEN_LED, OUTPUT);

pinMode(RED_LED, OUTPUT);

Serial.begin(9600);

Serial.println("--------------------------------");

Serial.println("Watchdog Timer Demonstration");

Serial.println("System Boot Successful");

Serial.println("--------------------------------");

delay(1000);

startTime = millis();

// Enable Watchdog (2-second timeout)

wdt_enable(WDTO_2S);

}

void loop() {

// Normal operation

digitalWrite(GREEN_LED, HIGH);

digitalWrite(RED_LED, LOW);

delay(300);

digitalWrite(GREEN_LED, LOW);

delay(300);

Serial.println("System Running...");

Serial.println("Watchdog Reset");

// Feed the Watchdog

wdt_reset();

// Simulate failure after 10 seconds

if ((millis() - startTime > 10000) && !failureTriggered) {

failureTriggered = true;

Serial.println("");

Serial.println("***********************************");

Serial.println("ERROR DETECTED!");

Serial.println("Sensor Communication Failure");

Serial.println("CPU HANG SIMULATED");

Serial.println("Watchdog will reset the system...");

Serial.println("***********************************");

digitalWrite(GREEN_LED, LOW);

digitalWrite(RED_LED, HIGH);

// No watchdog reset during this delay.

// Since watchdog timeout = 2 seconds,

// Arduino will automatically restart.

delay(5000);

}

}

SIMULATION :-